Supplemental Digital Content 3 - Physiologic waveform feature extraction *

| Physiologic waveform signal | Signal processing method | Features extracted | Taut string є values* |
| --- | --- | --- | --- |
| Electrocardiogram (ECG) Lead II, 240 Hz sampling rate | Peak detection and taut string estimation of heart rate variability | 6 features: - Number of line segments per heartbeat - Number of inflection segments per heartbeat - Total variation of noise per heartbeat - Total variation of denoised signal per heartbeat - Power of denoised signal - Power of noise | 0.0010 0.0258 0.0505 0.0753 0.1000 |
|  | Peak detection and taut string estimation | (same as row above, 6 features) | 0.0100 0.1575 0.3050 0.4525 0.6000 |
|  | Dual-tree complex wavelet packet transform of taut string estimation | 19 features per filter bank (2k) per wavelet transform (2), 152 total: - Standard deviation - Shannon entropy - Log energy entropy - Energy - Power - Minimum - Mean - Maximum - Median of largest 16 - Range - Mean gradient - Kurtosis - Skewness - Complexity - Mobility - Log of variance of probability distribution - Mean value of amplitude of fast Fourier transform - Sum of auto-correlation sequence - Mean value of cross-covariance | N/A; (k = 2) |
| Arterial blood pressure (ABP), 120 Hz sampling rate | Peak detection and taut string estimation | 21 features: - Total number of peaks (1) - Time interval between consecutive systolic peaks: minimum, median, mean, maximum, standard deviation (5) - Time interval between systolic peak and subsequent diastolic peak: minimum, median, mean, maximum, standard deviation (5) - Standard deviation of relative amplitude between consecutive systolic peaks: minimum, median, mean, maximum, standard deviation (5) - Standard deviation of relative amplitude between systolic peak and subsequent diastolic peak: minimum, median, mean, maximum, standard deviation (5) | 0.1 0.7 1.3 1.9 2.5 |
| Pulse plethysmography (SpO2), 60 Hz sampling rate | Peak detection and taut string estimation | (same as row above, 21 features) | 1 8.75 16.5 24.25 32 |

* Methods described in further detail in:

Hernandez L, Kim R, Tokcan N, Derksen H, Biesterveld BE, Croteau A, Williams AM, Mathis M, Najarian K, Gryak J. Multimodal tensor-based method for integrative and continuous patient monitoring during postoperative cardiac care. Artif Intell Med 2021;113:102032.